Florida International University

Project #3

PR Pico and LCD1602

EML 4804 Introduction to Mechatronics

Submitted By:

Elyion Clyburn

PID 6410764

Date of submission:

09/30/2025

Fall, 2025

Introduction

The goal of this project is to learn how to send messages from a microcontroller to a human operator using an LCD screen while gaining practical skills for working with more complex circuits. By programming the Raspberry Pi Pico to communicate with the LCD1602, I was able to practice writing code that controls hardware, and build experience troubleshooting communication errors between devices. This project also continued my self-guided work with embedded systems applications, where displaying information clearly and reliably is a fundamental task.

Materials used

- Breadboard

- Raspberry PICO board

- LCD screen

- Circuit wires

Picture

Code

import machine

import time

#Making the library

class LCD():

def __init__(self, addr=0x27, blen=1):

sda = machine.Pin(0)

scl = machine.Pin(1)

self.bus = machine.I2C(0,sda=sda, scl=scl, freq=400000)

#print(self.bus.scan())

self.addr = addr

self.blen = blen

self.send_command(0x33) # Must initialize to 8-line mode at first

time.sleep(0.005)

self.send_command(0x32) # Then initialize to 4-line mode

time.sleep(0.005)

self.send_command(0x28) # 2 Lines & 5*7 dots

time.sleep(0.005)

self.send_command(0x0C) # Enable display without cursor

time.sleep(0.005)

self.send_command(0x01) # Clear Screen

self.bus.writeto(self.addr, bytearray([0x08]))

def write_word(self, data):

temp = data

if self.blen == 1:

temp |= 0x08

else:

temp &= 0xF7

self.bus.writeto(self.addr, bytearray([temp]))

def send_command(self, cmd):

# Send bit7-4 firstly

buf = cmd & 0xF0

buf |= 0x04               # RS = 0, RW = 0, EN = 1

self.write_word(buf)

time.sleep(0.002)

buf &= 0xFB               # Make EN = 0

self.write_word(buf)

# Send bit3-0 secondly

buf = (cmd & 0x0F) << 4

buf |= 0x04               # RS = 0, RW = 0, EN = 1

self.write_word(buf)

time.sleep(0.002)

buf &= 0xFB               # Make EN = 0

self.write_word(buf)

def send_data(self, data):

# Send bit7-4 firstly

buf = data & 0xF0

buf |= 0x05               # RS = 1, RW = 0, EN = 1

self.write_word(buf)

time.sleep(0.002)

buf &= 0xFB               # Make EN = 0

self.write_word(buf)

# Send bit3-0 secondly

buf = (data & 0x0F) << 4

buf |= 0x05               # RS = 1, RW = 0, EN = 1

self.write_word(buf)

time.sleep(0.002)

buf &= 0xFB               # Make EN = 0

self.write_word(buf)

def clear(self):

self.send_command(0x01) # Clear Screen

def openlight(self):  # Enable the backlight

self.bus.writeto(self.addr,bytearray([0x08]))

# self.bus.close()

def write(self, x, y, str):

if x < 0:

x = 0

if x > 15:

x = 15

if y < 0:

y = 0

if y > 1:

y = 1

# Move cursor

addr = 0x80 + 0x40 * y + x

self.send_command(addr)

for chr in str:

self.send_data(ord(chr))

def message(self, text):

#print("message: %s"%text)

for char in text:

if char == '\n':

self.send_command(0xC0) # next line

else:

self.send_data(ord(char))

#Displaying my name on the LCD screen

from lcd1602 import LCD

import utime

lcd = LCD()

string = " Hello!\n"

lcd.message(string)

utime.sleep(2)

string = "    Sunfounder!"

lcd.message(string)

utime.sleep(2)

lcd.clear()

Video Link

https://youtu.be/aUo3NFlBaBA

Conclusion

- What have you learned?

From this project I have learned how to create a new symbol in KiCAD, and how to display text on the LCD screen.

- How difficult was this assignment?

This assignment was not difficult in concept, just difficult to get to work. I would give the difficulty a 5/10 because of this.

- How did you overcome challenges?

The only challenge I came across was the error message “Exception: No LCD found.” I decided to try a few different things in order to get it to work: I changed out the wires, unplugged and plugged back in all my wires, and I even had to go as far as deleting the “LCD1602.py” and “LCD.py” codes from my Raspberry Pico board. It wasn’t until I did the latter, then re-created the codes, that my LCD displayed my name as intended.

References

Home Project #3 PR Pico and LCD1602 Canvas Rubric

2.17 Show Characters on I2 LCD1602 — SunFounder Thales Kit for Raspberry Pi Pico 1.0 documentation

new symbol in KiCAD